JEL Gen. 5 MOCVD

( 1 TC, 3 PC & 2 L/L )

# SYSTEM Specification

Revision 6.5

2008.01.08

Ⅰ. Hardware Specification

1. Basic Configuration

  - Main Module

|  | 품 목 | 수 량 |
| --- | --- | --- |
| 1 | Transfer Chamber -.Possible to attach 3 PC & 2LL | 1 |
| 2 | Load Lock Chamber | 2 (Single slot) |
| 3 | Pre-heating Chamber | 2 (Single slot) |
| 4 | MOCVD Single substrate Process Chamber | 3 |
| 5 | Gas Rack | 1  3 |
| 6 | Remote Sub-Modules ( Sub Fab ) |  |
| 7 | 1) Main Power Rack | 1 |
|  | 2) Process Pump Module | 3 |
|  | 3) L/L & TC pump module | 2 |
|  | Scrubber System | 3 |

  - Glass Size : 1100mm × 1300mm

  - Glass Thickness : 3.1±0.2mm and 5±0.2mm, 4±0.2mm.

  - Film Deposition : ZnO (Hydorgen or Boron Doping)

  - Hardware Safety Interlock

1) Gas Interlock

2) RF Interlock

3) Hardware interlock

  - Jusung Supply Parts

- Heat Exchanger

- Dry pump system

- Scrubber

2. Transfer Chamber

2.1  Transfer Chamber

- Chamber Material : SUS304 Stainless Steel

- Lid Material : Al 6061

- Chamber Volume : 13,000liter

- Surface treatment : 내부 EP, 외부 Sanding EP

- Other Material exposed to vacuum : Viton, 강화유리

- Sealing : Viton O-ring

- View Ports on lid (Total 5EA) : ISO160

- Pumping Line : ISO 100 (Leak check port: NW25)

- Vent Kit (Total 2ea): 1/2” : GN2 (1st) -> CDA (2nd)

- N2 Purge with needle valve (Vent용)

- Leak Check Port (KF 25 ,with mannul valve , bottom of chamber )

  - Transfer Module Slit Valve

1) Load lock gate valve와 door valve

- Actuation Time : 3.5 <t< 4 sec

- Cycle life: >500,000cycle

- Open Size : 110mm(H)×1150mm(L)

- Qty : 6ea - ATM방향 2ea, Transfer Module방향 4ea

2) Process Module쪽 slit valve

- Actuation Time : 3.5 < t < 4 sec

- Cycle life: >500,000cycle

- Open Size : 110mm(H)×1150mm(L)

- Qty : 3ea – 각 Process module입구 마다 1ea씩

  - Transfer Module Frame

1) Conduction : Welded structural steel tubing

2) Surface treatment : Painting(N9.0 white, for electric charge protection)

3) Leveling Pad adjustment : ± 30mm

  - DLX Robot

  - Mount : From Top of chamber

  - R( Radial ) Axis

    - Position Repeatability :  							       < 0.7mm

    - Horizontal Trajectory Envelope :							  < 0.2mm

    - Vertical Trajectory Envelope :   						  < 8mm

    - Vertical Deflection :          							  < 4mm

    - Max Stroke( Robot Center to LL center ) :			2997mm

  - Theta( Rotational ) Axis

- Rotation :              											-185 ~ 140 degrees

- Position Repeatability :    										< 0.015 degrees

  - Z( Vertical ) Axis

- Stroke of Travel :       											670mm

- Position Repeatability :  											< 0.1mm

  - Thermal Exposure

- Drive Assembly :													    	80℃( Max )

- Arm :           								  						80℃( max & Operating temp )

- Wrist / End Effector :  												450℃( max & Operating temp )

  - Arm Assembly

- Arm style : Dual type

- End effector material :                    Ceramic, 2-Forks

- Substrate supports 재질 :                Perfluoelastomer or 그 호환품

- Substrate 형태 :                        4각 Pad (타사 호환가능)

  - Vacuum Level

  - Leak rate < 1*10-9 std.cc/sec He

  - Controls interface :RS-232 (DB9 Serial)

Dedicated serial port for hand held control display module

Miscellaneous parallel I/O ( 25 inputs and 18 outputs )

  - Power requirements

48VDC (500W Motor) (2x) 10A Peak/5A continuous and Z brake

  - TM Power Supply

- Input : 208VAC +/-10%, 50~60 Hz, 30A, 3Phase

and 110VAC / 220VAC +/-10%, 50~60 Hz, 2A, 1Phase

  - Vacuum Gauge

- Mini Convectron Gauge(Granville)

- ATM Switch (Working Range : -100 ~ 100kPa)

3.  Load Lock Chamber

3.1 Load Lock Chamber Module (2 chamber, Single Solt)

- Material : SUS304, Lid : Al 6061

- Substrates : 1100×1300mm

- Other Material exposed to vacuum : Viton, lexan

- Sealing : Viton O-ring

- Door open (Width×Height) : 110mm(H)×1150mm(L)

- View Ports : 4EA(2EA / each Ch) , Size : 437 *81

- Load lock Chamber Volume : 330 liter X 2EA

- Vent Kit : 1/2” Tube(GN2) with PV & FV

- 1/2” Tube(N2) with Regulator

- Diffuer ( Diameter 23mm, Laminar flow methode, 2EA / Each chamber)

- Pumping Line : ISO 100

10)  Leak check port : NW25 1EA/LL with Manual Valve( 2LL fore line 공통결합부)

3.2 Load Lock frame

- Surface Treatment : Paint ( N9.0 white, for electric charge protection)

- Leveling Pad adjustment : ±30mm

3.2 Vacuum Gauge

MKS Pirani gage : 1ea, ATM S/W : 1ea

3.4 N2 Gas Cooling

4. MOCVD Single Substrate Process Chamber

4.1 Chamber Module

- Material : Al6061 T6( Chamber body/Lid/Diffuser Cover/Showerhead )

- Deposition Film : ZnO (Hydorgen or Boron Doping)

- Process Parameter

- Temperature : RT - 300℃, unit 0.1℃

- Pressure : 0.5 ~ 5 Torr, unit 0.1 torr

- Electrode distance : 15 - 50mm, Gap unit : 0.1 mm

- RF Power : 0.5 ~ 6.5 KW , unit  0.1 KW .

- Gas : Within Max. Flow Rate of each MFC

- Time : Min. value sets as 1sec

- Edge Exclusion : 5.0±2mm (Shadow Frame), No back side deposition(Edge Frame 사용시)

- View Ports : Total 4EA

- Process position : 2EA( UV shielded )

- Load position : 2EA ( UV shielded )

- Pumping Line : NW200

- Leak check port : NW25 1EA with Manual Valve(Pumping line)

- Chamber Slit Valve Blade O-Ring : Perfluor(GA65) or 그 호환품

4.2  Chamber frame

- Surface treatment : Paint ( N9.0 white, for electric charge protection)

- Leveling possible : Frame고정 type

4.3  Temperature Control Unit

- Susceptor

- Material : AL6061

- Type : Molding type의 Sheath Heater

- Control zone : 2 zone,

- Heater Capacity : 10 kw( Inner ), 10 kw(outer)

- T/C type : K-Type, dual type( Susceptor temp control & monitor)

- Operating Temperature Range : RT~ 300℃

- Supply Power : 440VAC

- Temp Uniformity : ＜ ±5℃ (@ Deposition Process Temperature/

Ar 5,000 sccm/1Torr/Glass T/C 20point, EE15mm/Spacing : 25mm)

- Ramping Rate :< 1.5℃/min

- Rotating Injector

Material : SUS316L

4.4  Compumotor(Motor)

- Motor : Nidec-Simpo co.(Yaskawa Series)

- Drive : Yaskawa Series

4.5  Matching Box

- Maker : MKS

- Model : MWH100

- RF Frequency & Power: 13.56MHz +/-0.005%,

- Maximum Power Rating : 50-6,500 W

- Maximum Current Rating : 120 Arms

- Voltage Rating : 4kVpk

4.6  RF Generator

- Maker : MKS

- Model : SurePower

- RF Frequency & Power : 13.56MHz, 6,500W

- Maximum Current Rating : 120 Arms

4.7  Vacuum System Configuration

- Vacuum Valve

- Gate Valve : VAT 일체형

- Throttle Valve : VAT 일체형

- Iso V/V for Baratron : Pneumatic Valve

- Iso V/V for Mini Convectron(Vaccume Line & Chamber) : Pneumatic Vavle

- Slow Pneumatic Valve

- Vacuum Gauge & Switch

- Mini Convectron Gauge (Chamber): Granville

- Baratron Gauge: MKS 628B type

- Mini Convectron Gauge(pumping line) : Granville

- 1ATM Switch

- 10Torr Switch : MKS

4.8  Controller

- Vacuum controller

Valve & Controller 일체형

- Temp Controller

WATLOW Temp Controller CLS208 (Temp control & Monitoring ) 1set

SCR(VSFP, 40A,60A/440VAC)

4.9   Device_Net I/O, RS485

4.10  PM Power BOX

Circuit Breaker : Each Control Unit

5.  Control System ( All related PC parts are availbale after 5 years or more )

5.1 Cluster Tool Controller

- CPU : PentiumⅣ 2.8 GHz, Window XP Profesional Base

- Main Memory : 1G

- Video Splitter

- Ethernet Hub:3COM Ethernet Hub 12Port(10/100)

- LCD Monitor : 2 ea (17” with Mouse Control )

- Muti ComputSwitch

- Hard Disk

5.2  Transfer Module Controller

- Pentium IV 2.8 GHz, Window XP Profesional Base

- Control Substrate Transport

- AC Power Distribution System

- Gauge Signal Controller

- AC Interlock Panel with EMO Circuit

- Device Net Card

- DeviceNet DIO, RS-485,RS232

- Data Gate way

- Transfer Module Sofware

- Hard Disk

5.3  Process Module Controller

- Pentium IV 2.8 GHz, Window XP Profesional Base

- Controlling Temperature, Pressure, Chuck Moving & RF Power

- Device Net DIO, RS485

- Device Net Card

- Data Gate way

- Process Module Sofware

- Hard Disk

5.4  Factory Automation Software

- SECSⅡ, HSMS Support

- Full Object Support for SECS

- Message

6.  Gas Rack(Gas Delivery System)

6.1  Gas Rack position : Separated Rack

6.2  Gas Rack Configuration (stick type)

- Manual Valve : KITZ

- Pneumatic Valve : KITZ

- Regulator : Parker

- Filter : Purer

- Digital Meter : Asahi-keiki

- Pressure Transducer : NAGANO

- PT Sensor Display Unit : psi

- MFC : AERA

| Gas No. | Gas 명 | 용도 | 유량 | Remark |
| --- | --- | --- | --- | --- |
| Gas1 | Ar | Curtain Purge | 3 liters | 1/4” VCR |
| Gas2 | Ar | Seal Purge | 1 liters | 1/4” VCR |
| Gas3 | H2 | Main Purge | 10 liters | 1/4” VCR |
| Gas4 | H2 | Dilute | 5 liters | 1/4” VCR |
| Gas5 | Ar | H2O Carrier | 3 liters | 1/4” VCR |
| Gas6 | Ar | DEZ Carrier | 3 liters | 1/4” VCR |
| Gas7 | 0.5 % B2H6/H2 | Dopant | 1 liters | 1/4” VCR |
| Gas8 | H2 | Dilute | 5 liters | 1/4” VCR |

7.  Source Rack (Source Delivery System)

7.1  Gas Rack position : Separated Rack

7.2  Gas Rack Configuration (stick type)

- 1 Manual Valve : KITZ

- Pneumatic Valve : KITZ

- EPC (Electronic Pressure Controller : STEC) or Throttle V/V(MKS)

- Source Canister Volume

-. DEZ : 10 liters

-. H2O : 6 liters

8.  Control Rack

8.1  CTC Computer

8.2  LCD Monitor

8.3  ICP,HUB,Data Gate Way

8.4  EMO Switch

8.5  Signal Tower

8.6  Multi-Consent

9.  Main Power Rack

9.1  Main Power from Facility : 공장측에서 제공

9.2  UPS Power from Facility : 공장측에서 제공

9.3  EMO Switch

9.4  Separate Power Distribution & Circuit Breaker for each module

9.5  RF Generator (6.5KW)

10.  Process Pump Module (3 ea)

10.1  Process Chamber Dry Pump for each PM

Recommendation

Pumping Speed : > 60,000 Liters/min

Supply Voltage : 440V

Vacuum Inlet Connection for each pump : 200mm *2 ea

Exhaust flexible outlet for each pump : 40mm * 1 ea

  - Pump Rack ( 1 Pump Rack per Process Pump)

Recommendation

N2 flow meter

N2 pressure switch

Over pressure switch

Exhaust flow meter

11.  Load Lock & Transfer Cahmber Pump

( 2ea for Load lock & transfer chamber)

11.1  Load Lock & Transfer Chamber Dry Pump

Recommendation

Pumping Speed : > 30,000 Liters/min

Supply Voltage : 440V

Vacuum Inlet Connection for each pump : 160mm * 1 ea

Exhaust flexible outlet for each pump: 40mm * 1 ea

12.  Crane (KISCO will supply : 5ton)

  - Crane Type : Bridge type

12.2  Process Chamber Lid Weight : < 1900 Kg

- PM Operation tool  (JEL will supply)

  - Thermocouple glass (20 Point)

  - Spacing tool

  - Maintenance Jig

- PC Lid Rotation Jig										1 ea

- Susceptor & Showerhead Lift Jig			1 ea

- Susceptor Positioning Jig							2 ea

- TC Glass Lift Jig											1 ea

- Maintenance Footing									2 ea

- Susceptor Leveling (Spacing) Tool			1 ea (중복)

- Slot Valve Maint Jig										1 ea

14.  Scrubber (Burn adsorption & Wet) (JEL will supply)

- 처리 용량 : 500 SLM

- Power : A/C 200 ~240V, 3상, 15kW, 75A(max.)

- 세정가능가스 : H2, H2O, DEZ, TMA

- Gas inlet : NW50 Flange 4ports

- Gas Exhaust : NW100 Flange, -50 ~ -100mmH2O

15. Manuals (JEL will provide below manuals)

  - Operation Manuals: updated version for 5th G.

  - Maintenance Manuals, a menu for maintenance only.

  - facility Manuals , a Manuals for the facility only , also KISCO asks our file must be detail  with drawing with power , gas , water line

  - Electric circuit, a Manual for the detail tool’s electric circuit.

  - Main part’s explosion pictures, a Manuals for detail pictures to indicate the structure.

  - Spare parts lists, a Manual contain parts name and parts ID for KISCO to purchase our parts in the future.

  - OEM Manuals

18. Warranty

- AT Pass 또는 System Shipping 후 90일중 빠른 기간을 Start 시점으로 한다.

- Warranty 기간은 Start 후 1년

- Consumable Parts 제외

19. Consumable Parts List (TBD)

- O-ring 류

- Gasket 류

- Bolt류

- Ground Strip

- Robot Arm Pad

- Rotating Injector

- Golf tee, Golf tee bracket, Golf tee bushing, Golf tee pad류

- Inner jacket (방착판)

** TBD(To Be Determined)항목은 중간 검수 전에 결정함

Documentation

Manuals :

Two copies of each manual listed below on regular paper and clean paper are provided when these modules are part of the system.

Robot manual(CD)

System manual(CD, Hardcopy 2 books)

Drawings :

The following drawings are provided in each system manual:

System outline drawing for installation showing foot position, utilities hook up.

System schematic diagram included:

Vacuum diagram

Pneumatic diagram

PCW diagram

System interface drawings

** This specification is subject to be changed during the discussion and both parties should inform the other party if changes occurred and make an amendment.

- End of Document -