JEL Gen. 5 PECVD SYSTEM

( 1 TC, 5 PC & 2 L/L with Single Slot )

# SYSTEM Specification

Revision 6.6

2008.  01. 08

Ⅰ. Hardware Specification

1. Basic Configuration

  - Main Module

|  | 품 목 | 수 량 |
| --- | --- | --- |
| 1 | Transfer Chamber -.Possible to attach 6 PC & 1LL, Preheat chamber1 | 1 |
| 2 | Load Lock Chamber | 1(Single slot) |
| 3 | Pre-Heating Chamber | 1(Single slot) |
| 4 | PECVD Single substrate Process Chamber | 5 |
| 5 | Gas Rack | 1 |
| 6 | Control Rack | 1 |
| 7 | Remote Sub-Modules ( Sub Fab ) |  |
|  | 1) Main Power Rack | 1 |
|  | 2) Process Pump Module | 5 |
|  | 3) L/L & TC pump module | 2 |
| 8 | Scrubber System | 5 |

  - Glass Size : 1100mm × 1300mm

  - Glass Thickness : 3.1±0.2mm and 5±0.2mm, 4±0.2mm

  - Film Deposition : P, I & N  a-Si:H film (P, N μc-Si:H)

  - Hardware Safety Interlock

1)Gas Interlock

2)RF Interlock

3)Hardware interlock

  - Jusung Supply Parts

- Dry pump system

- Scrubber

2. Transfer Chamber

2.1  Transfer Chamber

- Chamber Material : SUS304 Stainless Steel

- Lid Material : Al 6061

- Chamber Volume : 13,000liter

- Surface treatment : 내부 EP, 외부 Sanding후 EP

- Other Material exposed to vacuum : Viton, 강화유리

- Sealing : Viton O-ring

- View Ports on lid (Total 7EA) : ISO160

- Pumping Line : ISO 100 (Leak check port: NW25)

- Vent Kit (Total 4ea): 1/2” : GN2 (1st) -> CDA (2nd)

- N2 Purge with needle valve (Vent용)

- Leak Check Port (KF 25 ,with mannul valve , bottom of chamber)

- TMP 장착이 가능한 Ports구성

  - Transfer Module Slit Valve

1) Load lock gate valve와 door valve

- Actuation Time : 3.5 <t< 4 sec

- Cycle life: >500,000cycle

- Open Size : 110mm(H)×1150mm(L)

- Qty : 4ea - ATM방향 2ea, Transfer Module방향 2ea

2) Process Module쪽 slit valve

- Actuation Time : 3.5 < t < 4 sec

- Cycle life: >500,000cycle

- Open Size : 110mm(H)×1150mm(L)

- Qty : 5ea – 각 Process module입구 마다 1ea씩

  - Transfer Module Frame

1) Conduction : Welded structural steel tubing

2) Surface treatment : Painting(N9.0 white, for electric charge protection)

3) Leveling Pad adjustment : ± 30mm

  - DLX Robot

  - Mount : From Top of chamber

  - R( Radial ) Axis

    - Position Repeatability  							      :  < 0.7mm

    - Horizontal Trajectory Envelope :							  < 0.2mm

    - Vertical Trajectory Envelope   :						  < 8mm

    - Vertical Deflection           :							  < 4mm

    - Max Stroke( Robot Center to LL center ) :			 2997mm

  - Theta( Rotational ) Axis

- Rotation             : 												-185 ~ 140 degrees

- Position Repeatability  :  											< 0.015 degrees

  - Z( Vertical ) Axis

- Stroke of Travel       : 												 670mm

- Position Repeatability  :												 < 0.1mm

  - Thermal Exposure

- Drive Assembly :																		 80℃( Max )

- Arm          : 																		80℃( max & Operating temp )

- Wrist / End Effector   														: 450℃( max & Operating temp )

  - Arm Assembly

- Arm style : Dual type

- End effector material :  Ceramic, 2-Forks

- Substrate supports 재질: Perfluoelastomer

- Substrate 형태 : 4각 Pad (타사 호환가능)

  - Vacuum Level

  - Leak rate < 1*10-9 std.cc/sec He

  - Controls interface :RS-232 (DB9 Serial)

Dedicated serial port for hand held control display module

Miscellaneous parallel I/O ( 25 inputs and 18 outputs )

  - Power requirements

48VDC (500W Motor) (2x) 10A Peak/5A continuous and Z brake

  - TM Power Supply

- Input : 208VAC +/-10%, 50~60 Hz, 30A, 3Phase

and 110VAC / 220VAC +/-10%, 50~60 Hz, 2A, 1Phase

  - Vacuum Gauge

- Convectron Gauge(Granville)

- Baratron Gauge(MKS)-향후 필요시 port만 유지

- ATM Switch (Working Range : -100 ~ 100kPa)

3.  Load Lock Chamber

3.1  Load Lock  Chamber Module( 1 chamber, Single Slot)

- Material : SUS 304, Lid : AL6061

- Substrates : 1100×1300mm

- Other Material exposed to vacuum : Viton, Lexan

- Sealing : Viton O-ring

- Door open (Width×Height) : 110mm(H)×1150mm(L)

- View Ports : 4EA(2EA / each Ch) , Size : 437 *81

- Load lock Chamber Volume : 300 liter X 2EA

- Vent Kit : 1/2” Tube(GN2) with PV & FV

- 1/4” Tube(He) with Regulator

- Diffuser ( Diameter 23mm, Laminar flow methode, 1EA / Each chamber)

- Pumping Line : ISO 100

- Leak check port : NW25 2EA with Manual Valve( 2LL fore line 공통결합부)

  - Load Lock frame

- Surface Treatment : Paint ( N9.0 white, for electric charge protection)

- Leveling Pad adjustment : ±30mm

3.3  Vacuum Gauge

MKS Baratron Gauge (MKS 722 ) : 10Torr range 1EA, ATM switch 1EA

3.4  N2 Gas Cooling

4. PECVD Single Substrate Process Chamber

4.1 Chamber Module

- Material : Al6061 T6( Chamber body/Lid/Diffuser Cover/Showerhead )

- Deposition Film : P, I & N  a-Si:H film (P μc-Si:H)

- Process Parameter

- Temperature : RT - 300℃, unit 0.1℃

- Pressure    : 0.5 ~ 5 Torr, unit 0.1 torr

- Electrode distance : 15 - 40mm, Gap unit : 0.1 mm

- RF Power  : 0.5 ~ 5 KW , unit  0.1 KW .

- Gas       : Within Maximum Flow Rate of each MFC

- Time      : Min value sets as 1sec

- Edge Exclusion : 5.0±2mm (Shadow Frame), No back side deposition

- View Ports : Total 4EA

- Process position : 2EA( UV shielded )

- Load position : 2EA ( UV shielded )

- Pumping Line : NW200

- Leak check port : NW25 2EA with Manual Valve(Pumping line & Bottom of Ch.)

- Chamber Slit Valve Blade O-Ring : Perfluor(GA65)

4.2  Chamber frame

- Surface treatment : Paint ( N9.0 white, for electric charge protection)

- Leveling possible : Frame고정 type

4.3  Temperature Control Unit

- Susceptor

- Material        : AL6061

- Type           : Molding type의 Sheath Heater

- Control zone    : 2 zone,

- Heater Capacity : 10 kw( Inner ), 10 kw(outer)

- T/C type       : K-Type, dual type( Susceptor temp control & monitor)

- Operating Temperature Range : RT~ 300℃

- Supply Power   : 440VAC

- Temp Uniformity : ＜ ±5℃ (at process deposition temperature /N2 10,000scc, 1 Torr/Glass T/C 20point, EE15mm/Spacing : 25mm)

- Ramping Rate :< 1.5℃/min

- Showerhead head

Material : AL6061

4.4  Compumotor(Motor)

- Motor : Yaskawa Series

- Drive : Yaskawa Series

4.5  Remote Plasma Source Cleaning(1EA/Chamber)

- NF3 Cleaning

- Maker : MKS

- Model : AX7685-01( NF3 7.5liter)

4.6  Matching Box

- Maker : MKS

- Model : MWH100

- RF Frequency & Power: 13.56MHz +/-0.005%,

- Maximum Power Rating : 50-5,000 W

- Maximum Current Rating : 120 Arms

- Voltage Rating : 4kVpk

4.7  RF Generator

- Maker : MKS

- Model : SurePower 6.5kW

- RF Frequency & Power : 13.56MHz, 5,000W

- Maximum Current Rating : 120 Arms

4.8  Vacuum System Configuration

- Vacuum Valve

- Gate Valve : VAT 일체형

- Throttle Valve : VAT 일체형

- Iso V/V for Baratron : Pneumatic Valve

- Iso V/V for Convectron(Vaccume Line & Chamber) : Pneumatic Vavle

- Iso V/V for 10Torr Switch : Pneumatic Vavle

- Slow Pneumatic Valve

- Vacuum Gauge & Switch

- Convectron Gauge : Granville

- Baratron Gauge: MKS 628B type

- Mini Convectron Gauge(pumping line) : Granville

- 1ATM Switch

- 10Torr Switch : MKS

- Cold Cathode Gauge

3 )  Turbo Molcular Pump

- Pumping Speed : 2700 liter /min, STP XA 2703CU

- Isolation valve : ISO250, VAT 12148-PA44-AGW1

4.9  Controller

- Vacuum controller

      - Valve & Controller 일체형

- Temp Controller

WATLOW Temp Controller CLS208 (Temp control & Monitoring ) 1set

SCR(VSFP, 40A,60A/440VAC)

4.10  Device_Net I/O, RS485

4.11  PM Power BOX

Circuit Breaker : Each Control Unit

5.  Control System ( All related PC parts are availbale after 5 years or more )

  - Cluster Tool Controller

- CPU : PentiumⅣ 2.8 GHz, Window XP Profesional Base

- Main Memory : 1G

- Video Splitter

- Ethernet Hub:3COM Ethernet Hub 12Port(10/100)

- LCD Monitor : 2 ea (17” with Mouse Control )

- Muti ComputSwitch

- Hard Disk

5.2  Transfer Module Controller

- Pentium IV 2.8 GHz, Window XP Profesional Base

- Control Substrate Transport

- AC Power Distribution System

- Gauge Signal Controller

- AC Interlock Panel with EMO Circuit

- Device Net Card

- DeviceNet DIO, RS-485,RS232

- Data Gate way

- Transfer Module Sofware

- Hard Disk

5.3  Process Module Controller

- Pentium IV 2.8 GHz, Window XP Profesional Base

- Controlling Temperature, Pressure, Chuck Moving & RF Power,RPSC

- Device Net DIO, RS485

- Device Net Card

- Data Gate way

- Process Module Sofware

- Hard Disk

5.4  Factory Automation Software

- SECSⅡ, HSMS Support

- Full Object Support for SECS

- Message

6.  Gas Rack(Gas Delivery System)

6.1  Gas Rack position : Separated Rack (Stand alone type)

6.2  Gas Rack Configuration (stick type)

- Manual Valve : KITZ

- Pneumatic Valve : KITZ

- Regulator : VERIFLO

- Filter : KITZ

- Digital Meter : Asahi-keiki

- Pressure Transducer : NAGANO

- PT Sensor Display Unit : psi

- MFC : AERA

| Gas No. | Gas 명 | 유량 | Remark |
| --- | --- | --- | --- |
| Gas0 | N2 | NA | 1/4” VCR |
| Gas1 | NF3 | 12 liters | 1/4” VCR |
| Gas2 | Ar | 12 liters | 1/4” VCR |
| Gas4 | Ar | Line Purge용 | 1/4” VCR |
| Gas5 | CH4 | 10 liters | 1/4” VCR |
| Gas6 | B2H6 | 10 liters | 1/4” VCR |
| Gas7 | SiH4 | 5 liters | 1/4” VCR |
| Gas8 | PH3 | 10 liters | 1/4” VCR |
| Gas9 | H2 | 30 liters |  |

7.  Control Rack

7.1  CTC Computer

7.2  LCD Monitor

7.3  ICP,HUB,Data Gate Way

7.4  EMO Switch

7.5  Signal Tower

7.6  Multi-Consent

8.  Main Power Rack

8.1  Main Power from Facility : 공장측에서 제공

8.2  UPS Power from Facility : 공장측에서 제공

8.3  EMO Switch

8.4  Separate Power Distribution & Circuit Breaker for each module

8.5  Transformer

PRI 208VAC / SEC 208Y / 120VAC

9.  RF Generator rack

9.1  Type : Rack integration : 6.5KW

9.2  Cooling  : Water cooled

10.  Process Pump Module ( 5 ea)

10.1  Process Chamber Dry Pump for each PM  ( > 30,000Liter)

Maker : 성원 에드워드

Model : IH1800

Supply Voltage : 440V

Vacuum Inlet Connection for each pump : 200mm *1 ea

Exhaust flexible outlet for each pump : 40mm * 1 ea

  - Pump Rack ( 1 Pump Rack per Process Pump)

N2 flow meter

N2 pressure switch

Over pressure switch

Exhaust flow meter

11.  Load Lock & Transfer Pump

11.1  Load Lock Dry Pump for 1 Loadlock, Pre-heat chamber( > 30,000Liter)

Maker : 성원 에드워드

Model : DiF30KMM

Supply Voltage : 440V

Vacuum Inlet Connection for each pump : 160mm * 1 ea

Exhaust flexible outlet for each pump: 40mm * 1 ea

11.2 Transfer Chamber Dry Pump

Maker :성원 에드워드

Model : IH1800

Supply Voltage : 440V

Vacuum Inlet Connection for each pump : 200mm *1 ea

Exhaust flexible outlet for each pump : 40mm * 1 ea

- Crane (KISCO will Supply 5ton)

  - Crane Type : Bridge type

  - Process Chamber Lid Weight : 1900 Kg

- PM Operation tool  (JEL will supply)

  - Thermocouple glass (20 Point)

  - Spacing tool

  - Maintenance Jig

- PC Lid Rotation Jig										1 ea

- Susceptor & Showerhead Lift Jig			1 ea

- Susceptor Positioning Jig							2 ea

- TC Glass Lift Jig											1 ea

- Maintenance Footing									2 ea

- Susceptor Leveling (Spacing) Tool			1 ea

- Slot Valve Maint Jig										1 ea

14.  Scrubber (Burn adsorption & Wet) (JEL will supply)

- 처리 용량 : 500 SLM

- Power : A/C 200 ~240V, 3상, 15kW, 75A(max.)

- 세정가능가스 : SiH4, B2H6, PH3, CH4, H2, NF3(Dry & Wet type)

- Gas inlet : NW50 Flange 4ports

- Gas Exhaust : NW100 Flange, -50 ~ -100mmH2O

- Manuals (JEL will provide below manuals)

  - Operation Manuals: updated version for 5th G.

  - Maintenance Manuals, a menu for maintenance only.

  - facility Manuals , a Manuals for the facility only , also KISCO asks our file must be detail with drawing with power , gas , water line

  - Electric circuit, a Manual for the detail tool’s electric circuit.

  - Main part’s explosion pictures, a Manuals for detail pictures to indicate the structure.

  - Spare parts lists, a Manual contain parts name and parts ID for KISCO to purchase our parts in the future.

  - OEM Manuals

- Warranty

  - AT Pass 또는 System Shipping 후 90일중 빠른 기간을 Start 시점으로 한다.

  - Warranty 기간은 Start 후 1년

  - Consumable Parts 제외

- Consumable Parts List( TBD)

- O-ring 류

- Gasket 류

- Bolt류

- Ground Strip

- Robot Arm Pad

- Golf tee와 Golf tee bracket, Golf tee bushing, Golf tee pad류

** TBD(To Be Determined)항목은 중간 검수 전에 결정함.

Documentation

Manuals :

Two copies of each manual listed below on regular paper and clean paper are provided when these modules are part of the system.

Robot manual(CD)

System manual(CD, Hardcopy 2 books)

Drawings :

The following drawings are provided in each system manual:

System outline drawing for installation showing foot position, utilities hook up.

System schematic diagram included:

Vacuum diagram

Pneumatic diagram

PCW diagram

System interface drawings

** This specification is subject to be changed during the discussion and both parties should inform the other party if changes occurred and make an amendment.

- End of Document -